| Departmental Hazards & Risks Register: Brief description of work-related hazards/risks and control. |  |  |  |
| --- | --- | --- | --- |
| Nature of Hazards | Hazards Source | Risk Level | Risk Control |
| Electricity (Electrocution, induction, Electric flash, Power line Contact) | Contact with live wire (33, 11 & 0.415) kV | High | Ensure Proper outage management, teasing of the line, Application of Temporary grounding, Working at safe distance from overhead wires (i.e. minimum distance of 1.0 meter for 33kv & 0.6 meter for 11kv), use of caution sins, use of PPE. Use of insulated tools, and use of approved company procedure. |
| Height Difference (Fall from height, Slip, trip, Object Falling from height) | Structure failure i.e pole collapsing, over stretching of Ladder, trenches/excavation working tools, equipment/conductor lifted | High | Ensure Physical inspection of the structure/Pole, avoid using Ladders with defects, Use of tool rag, Fall arrest equip / personnel guard/ harness Heavy or awkward load Climbing aids Slippery / uneven surface Harness Shock absorbing lanyard and hooks Stanchions, and PPE (i.e. Safety belt and helmet, safety boot) |
| Overhead Object Loss/Falling of load/Crane failure, Dropped loads | Rigging failure, excessive load lifting & sling wire, rob and chain failure | High | Do not exceed the crane lifting capacity, ensure loads shall be balance within it center of mass, Competent operator must be used, inspection of crane system, rob/chain, Keep away from crane radius operation, & Use tag lines on all lifts |
| Biological Medium (Poisoning, biting) | Reptiles, Rodents and insects attack | Medium | Health education, Pest control / fumigation, Use Appropriate PPE Health surveillance |
| Hazardous substances (Contact with chemical substances) | PCB Contained transformer oil, Petroleum solvent and other chemical substances. | High | Use of PPE (Safety boot, Coverall, rubber Hand gloves), Face mask, proper outage management and ensure the application on cold surfaces. |
| Weather Condition (Lightening, heavy wind, rainfall) | Storm, extreme temperature | Low | Suspension (no work in adverse weather / lighting), Adequate earthing Toolbox meeting |
| Violent Crime (Assault, Violence against individual) | Contact with aggrieved and unsatisfied customers, Breakdown of social order, lawlessness, riot, absence of government authority, Pressure Groups, violent or threatening protests against Company, people or asset | Medium | Ensure professionality while dealing with customers, follow Emergency Response Procedures, use negotiation skills and Counseling tactics, ensure Standby security personnel and Post event dialogue |
| Job Demand (Excessive work load) | Poor job planning and inadequacy of Manpower, frequent deadlines, and poor work-life balance, | Medium | Adequate planning of the job, Utilization of annual leave, training, and good designed shift patterns or rosters. |
| NOTE: Hard hats, and safety footwear are required in all Kaduna Electric field locations (Mandatory) |  |  |  |

KADUNA ELECTRIC EMPLOYEE OCCUPATIONAL HEALTH AND SAFETY RISK ATTESTATION FORM

Note: This document will be reviewed periodically to reinforce safety awareness and ensure ongoing compliance.